| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage, Back Ream) | 51,10, 5 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 5 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 19.04 |  |
| Max. TFA (Sq. Inches) | 1.86 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 16,488 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 15.55 |  |
| Make Up Length (Inches) | 11.49 |  |
| Bit Breaker | SKB-21010 |  |
| Connection | Ø4-1/2" API Reg. |  |
| Approx. Weight (Lbs.) | 110 |  |

Standard Features:

Large open face volume assists cuttings removal

Seven nozzles optimize blade cleaning and cuttings removal

Premium grade cutters maximize ROP and Life

Selectively positioned bullets for smooth drilling

1" full gage, 2" tapered gage at 1°

516-XR(K)(BR) Nomenclature Explained:

| - • | - 5 | - = | - Number of Blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - XR+ | - = | - Backup Cutters |
| - • | - K | - = | - Bullets |
| - • | - BR | - = | - Back Reaming Cutters |