2177 Ø12-1/4” 616-XR9(K)(BR)

2177 has a backup layout similar to 2138, including the .4in/rev placement, but on a more traditional profile (2138 was elliptical). Primary cutters have slightly (1/64th) more aggressive spacing in the cone, and a F63 layout.

- F63 cutter layout

- Carbide bullets just above the gage.

- Trimmers set at a durable backrake, aggressive backrake elsewhere.

- 6” Gage (2-1/2” at gage, 3-1/2 at 0.3° taper).

- Backup cutters underexposed .4"/rev, with a 10° more durable backrake.

- PDC bullets will be underexposed in the cone (.025”).

Features:

- F63 is new cutter layout that has a cutlet plot similar to a 6-3 design, but at lower ROP it looks like a traditional 6 pattern.

Bit Body:

Ø12-1/4”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

6” Gage, 2-1/2 @ gage, 3-1/2 @ 0.3° taper

Open Face Volume = 59.215 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.264" | 1.674" | .75 |
| Center of round | 1.657" | 1.980” | .83 |
| Gauge | 1.695" | 2.046” | .82 |

Primary Cutting Structure:

Non-redundant; Forward Spiral; Secondary blades are flush to primary; Knuckles are under exposed in the cone and nose at .025”; all other backup elements are under exposed @ .4”/rev.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	16-30°

Gage		35°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

50 - 1613 Primary Cutters

30 - 1313 Cutters

12 - ½” PDC Knuckles

18 - ½” TCI Knuckles