2179 Ø12-1/4” 716-XR(K)(BR)

2179 Uses the backrake scheme of the 2177 with a traditional reverse spiral non-redundant 7 blade design. Normal profile (non-ellipse). Going after the 2138 interval with the intent to increase the TD rate while not slowing down too much. Depth in at 1,600’, with depth out at 10,500’ on a motor. Customers include Callon, Elevation, Fasken, and Endeavor.

- Carbide bullets just above the gage.

- Trimmers set at a durable backrake and fades to aggressive backrake on inner shoulder.

- 6” Gage (2-1/2” at gage, 3-1/2 at 0.3° taper).

- Backup cutters underexposed .4"/rev, with a 10° more durable backrake.

Features:

- 7 blades to increase durability, and less aggressive backrake on outer shoulder to help with lateral vibrations.

Bit Body:

Ø12-1/4”

7 Blades

3 primary blades

4 secondary blades

52/44/46/54/58/50/56 Blade angles

15° Cone

6” Gage, 2-1/2 @ gage, 3-1/2 @ 0.3° taper

Open Face Volume = 53.427 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.264” | 1.553” | 0.814 |
| Center of round | 1.657” | 2.069” | 0.801 |
| Gauge | 1.695” | 2.236” | 0.758 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Backup elements are under exposed @ .4”/rev.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	16-30°

Gage		35°

Components:

07 - 22/32 Nozzle Sleeves

07 - 60 Series Nozzles

55 - 1613 Primary Cutters

42 - 1313 Cutters

21 - ½” TCI Knuckles