2180A Ø10-5/8” 716-XR(K)(BR)

2180 is based on 2145. DOC limiting TCI Bullets removed.  Backrake reduced in cone and nose to increase aggressiveness.  Cutter added in nose to increase durability.  Backups lowered to .6”/rev. Blade relief in cone.

For XTO. For use in LHS Ranch ≈1,200’ to 8,800’.  220 rpm, 660 ROP, 0.600 DOC

Bit Body:

Ø10-5/8”

7 Blades

3 primary blades

4 secondary blades

50/46/46/54/58/50/56 Blade angles

15° Cone

6” Gage; 2.5” @ full, 3.5” @ 0.3°

Open Face Volume = 33.639 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.226” | 1.443” | 0.850 |
| Center of round | 1.769” | 1.864” | 0.949 |
| Gauge | 1.371” | 2.121” | 0.646 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		17.5°

Nose 		16°

Shoulder 	16-30°

Gage		30°

Components:

07 - 22/32 Nozzle Sleeves

07 - 60 Series Nozzles

49 - 1613 Primary Cutters

21 - 1313 Cutters

12 - 1/2” TCI Bullets