2181A Ø7-7/8” 616-XR(K)(BR)

Center cutter and trimmers are 1613. Aggressive cone and nose with durable shoulder and gage. For XTO on bent motor (or FBH). Lateral, MD from 8000’ to 9350’, Wolfcamp A and B. Most offsets have 50’/hr overall average. Fastest are in low 60’s. Target is an overall of 60-70’/hr. 12.5° cone to allow steerability, but main target is stability as the ROP is much slower when sliding vs rotating (30 vs 150).

Features:

- Backups are set to engage at .25”/rev.

- 4” Gage with WP elements. 1.5” @ gage, with 2.5” @ 0.3°.

Bit Body:

Ø7-7/8”

6 Blades

3 primary blades

3 secondary blades

60/55/66/60/65/55 Blade angles

12.5° Cone

4” Gage, 1.5” @ gage, 2.5” @ 0.3°

Open Face Volume = 17.438 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.036” | 1.420” | 0.730 |
| Center of round | 1.080” | 1.902” | 0.568 |
| Gauge | 0.861” | 1.988” | 0.433 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

33 - 1613 Cutters (14 Saddle cutters)

16 - 1313 Cutters

06 - ½” TCI Bullets

18 - WP 1308