2185A Ø13-1/2” 616-SR+9(K)

Jim Montgomery (From Yammer, Apache Corporation):

Met with Tim Higgins this morning… He presented us with a TailorMade opportunity to design a 13-1/2" long gauge bit to drill to 3,200'-3,500' in the Delaware Basin, Reeves County. It will be run on a .16 motor with a 1.5 degree bend… The price for the run will be $13,500.

-The design will be a 2160 scaled down.

Features:

- Bullets on nose and outer shoulder for ROP limiters and impact arrestors.

- Three 1313 backups per blade.

- 7.00” gage

Bit Body:

Ø13-1/2”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

7.00” Gage

Open Face Volume = 99.373 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.676” | 1.504” | 1.114 |
| Center of round | 2.145” | 2.354” | 0.911 |
| Gauge | 1.688” | 2.604” | 0.648 |

Primary Cutting Structure:

Redundant; Reverse Spiral; Secondary blades are flush to primary; Backup cutters are set at .093”/rev +.05” under exposed. Bullets are set at .093”/rev +.075” under exposed

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

55 - 1613 Primary Cutters

30 - 1313 Cutters

24 - ½” TCI Knuckles