2186A Ø8-3/4” 616-XR(K)(BR)

2186 was designed for Mack Energy, San Andres wells. Made to drill the curve and lateral in one bit (TVD ≈ 3,357’. 8°/100’. ½” TCI in gage to help prevent gage wear. Durable cone and shoulder, with aggressive nose. Knuckles in cone and nose, backups on shoulder. Step profile to increase side cutting ability. 200’/hr, 195rpm = .205”/rev for backup placement.

Bit Body:

Ø8-3/4”

6 Blades

3 primary blades

3 secondary blades

60/60/58/62/55/65 Blade angles

9° Cone

3” Gage

Open Face Volume = 17.917 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.985 | 1.355 | .727 |
| Center of round | 1.582 | 1.774 | .892 |
| Gauge | 1.432 | 1.899 | .754 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		18°

Nose 		16°

Shoulder 	18-30°

Gage		35°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

30 - 1613 Cutters

24 - 1313 Cutters

06 - 3/8" TCI Bullets

06 - 1/2" TCI Bullets