1. Project Overview

- Mack Energy

- Chaves County, NM

- San Andres curve and Lateral

- .28 rev/gal @ 550 gal/min – 135-195 rpm

- Bent Motor

- Footage ≈ 6,100’

- 8°/100’

2. Performance Target

- U616M, BH 506 & 406 (no spec sheets or dull photos) (1 bit record, 3 bits to TD)

- Historical CapRock performance – N/A

- Target - 1 bit to TD

3. Design Concepts under consideration

- Backrake = 18° cone, 16° nose, taper to 30° Shoulder.

- 9° cone

- 616-XR, K (in cone), BR

- 3” Gage, 1.5” @ 0.3°

- Standard cutter spacing

4. Field Test Plan

- 1 bit (2-3 week of May 11th (1 week to design, 1 week to build)

- Currently have steel for the 2142 and new bit design.

- 2142 (shallow leach cutters)

- New design (≈ 30 1613 DP cutters)