Mack Energy Corporation

Thunder Bay Federal Com #1H (Horizontal)

San Andres

Chaves County, New Mexico

Mack Energy Corp AFE #: 4175.01H.				API #: 30-005-64308

Location: SHL 10’ FNL and 1650’ FWL / BHL 270’ FSL and 1675’ FWL in Sec 22-T15s-R29e

Chaves County, New Mexico.

Elevation: 3844’ GR     TDMD: 8,073.15’+ 80’ rat hole = 8,153’       TVD: 3,357’             KB:  17.5’

Rig Contractor:  Silver Oak Rig #11 Phone #: 575-513-0712       Tool Pushers:

Pit System:  Closed Loop.

Directions: From the intersection of State Hwy #82 and CR #217 (Hagerman Cutoff) Go North on CR #217 for 10.5 miles. Turn west on lease road (County Line Road) and go 2.1 miles continue northwest and go 2.4 miles to the South Lucky Queen 1E pad and west side of Hamilton Federal Com #1H pad. Then from northeast pad corner go southeast 880’ to the North pad corner for this location.

Anticipated Formation Tops / Logging and Coring Program

Top of Salt:            280’

Base of Salt:          885’                               KOP: 2,569’. Build rate 8 degrees/100’ AZI 178.83 degrees

Yates:                     1,050’                       End of build: MD 3,256.76’ TVD 3,155.94’ 55.00 degrees AZI 178.83

Seven Rivers:        1,050’                    Hold 55.00 Degrees AZI 178.83 Degrees until MD 3,456.76’ TVD 3,270

Queen:                   1,750’                       Begin 12 degree/ 100’ build at 3,456’. Landing point MD 3,748.43’

Grayburg:               2,145’                        TVD 3,357’ at 90.00 degrees AZI 178.84

San Andres:           2,445’ (Objective)       MDTD 8,073.15 TVD 3,357’ Vertical section 5,067.82’

Pilot Hole MDTD   3,600’ TVD 3,600’

MDTD: 8,153’ including 80’ of rat hole TVD 3,357’

Note: Set 40’ of 16” new conductor pipe.

Note: Possible loss circulation in surface section and possible air pocket in production section.

Mud Logs: Suttles logging. (432) 687-3148.  RIGGED UP and WORKING ON SURFACE SECTION.

Logging Company:  Schlumberger Midland Texas 432-742-5558

E-Log Suite:  Wire Line logs by Schlumberger (Pilot Hole)

Chemicals

- Run CAT 360 Corrosion/ Scale inhibitor 21 gallons per day starting on 1st joint of Drill pipe.

- Run 2) CAT 601 H2S Scavenger 21 gallons per day on 1st joint of Drill pipe.

Company: Catalyst                                                            Contact: Mike Jorren @ 575-513-0778

Note: IF the Mud logging equipment or the Gamma Ray is not working, Shut down and circulate until fixed. Do not drill any new hole without them.

Drilling Prognosis

| Section | Hole Size/ TD/ BHA | Drilling Fluids | Csg and Cement | Wellhead |
| --- | --- | --- | --- | --- |
| Surface Possible Lost Circulation and air pockets. Water flow | 14 3/4” TD @ 200’ Bit: Ulterra U616S, BH T606X or Equivalent BHA: Bit, Motor, SS, T-Drift w/AZM, RMR, 3-8" DC’s, X-O, 6" DC’s Consider running both pumps to drill this section. | Fresh Water MW: 8.3-8.6 Visc. 28-32 PH- 10 See attached Rec. | Casing 9 5/8"- 36#-J-55 ST&C Cement 50# PF-153 Gel Spacer. 50 sacks Scavenger Wt. 11# 100 sacks RFC Wt. 11#. Yld. 1.62 200 Sacks Class C + Additives. Wt. 14.8#. Yld. 1.33 TOC: Surface See Attached Rec. | 9 5/8” X 11”x 5K |
| Production Possible H2S and Air Pocket. Water flow | 8 3/4” Pilot hole TD @ 3,600’ Bit: Ulterra U616M, BH TD506FX or equivalent. BHA: Bit, Motor, SS, T-Drift w/AZM, RMR, 6" DC’s Consider running both pumps to drill this section. Log well with Schlumberger. Determine Kick off Plug | Cut Brine Water MW: 9.0-9.4 Visc.: 28 Water Loss:10cc – 12 cc at TD. See attached Rec. | Kick Off Plug 3,000’ – 2,000’ 400 sacks Class “H” with .50% PF-065 Density: 17.50# Yield .94 cuft./sk. Mix Water 3.365 gal/sk. See attached Rec. | 10K Tubing head |
|  | Dress off cement kick off plug with 8 3/4” bit to KOP. KOP 2,569.26’ MD 8 3/4” MDTD @ 8,231.47’ Bit: Ulterra U616M, BH TD406FX or equivalent. BHA: Bit and directional Tools to drill lateral. KOP 2,569.26’ MD Build Rate 8 degrees/100’ until MD 3,256.76’ TVD 3,155.94’ Azimuth 178.83 degrees. EOB 55 degree’s Hold 200’ Tangent from TVD 3,155.94’ to 3,270.65’ Continue Build Rate @ 12 degrees/ 100’. EOB 90 degrees MD 3,748.43’ TVD 3,357’ Hold 90 degrees until MD 8,073.15’ TVD 3,357’ AZM 178.84 Drill 80’ of rat hole past 8,073.15’. MDTD 8,153’ See attached Wellbore Plan #2 | Brine MW. 10 # Visc. 28-34 WL 10-20 cc PH: 11 Pump weighted sweep to clean hole as needed See attached Rec. | Casing 7”- 26#- P-110 LT&C from 0’ to 2,300’ Cross over to 7”-26#-P-110 BTC from 2,300’ to 3,050’ 5 ½”- 17# - P-110 BTC From 3,050’ to 8,074’ Cement 7”/ 5 ½” casing 50# PF-153 Gel Spacer. 20 Bbls. PF827 Mud Wash 50 sacks Scavenger Wt. 11# Lead Slurry: 375 Sacks Clss “C” + additives Wt. 13.2 Yld. 1.84 Tail Slurry: 1,155 sacks PVL + additives Wt. 13# Yld. 1.48 See attached Rec. | 10K Tubing head |

Drilling Fluids: Newpark Drilling Fluids Contact: Mike Davis. m.davis@newpark.com

Cell. # 432-557-8254

Cement Company: Par-Five Energy Services. Contact: Chris Deleon Office 575-748-8610 Cell 575-703-6723 or Kurt Crowe

Keep location and lease roads clean (Don’t Litter) if you see trash pick it up.

Casing

| Size | From | To | Weight lbs/ft | Grade | Thread |
| --- | --- | --- | --- | --- | --- |
| 9 5/8” | 0' | 200’ | 36# | J-55 | ST&C |
| 7” | 0’ | 2,300’ | 26# | P-110 | LT&C |
| 7” | 2,300’ | 3,050’ | 26# | P-110 | BTC |
| 5 ½” | 3,050’ | 8,153’ | 17# | P-110 | BTC |

Sandblasted pipe needed 8,153’ to 3,050’ on 5 ½” casing

Production casing Centralizer Program for 7” casing

3,050’- 2,200’ 1 centralizer every joint = 22.

2,200-100’1 centralizer every 3rd joint = 18.

Total = 40 centralizers.

Production casing Centralizer Program for 5 ½” casing

8,153’- 3,050’ 1 Solid Vane centralizer every 20’

Total = 256 Centralizers

Production casing:  5 ½”-17#-P-110 BTC Toe Sleeve: FS, Marker Joint, FC, Marker Joint, Debris Trap, Landing Collar, Toe Sleeve. 4 Bbls of sugar (retarded cement) cement from FS to Toe Sleeve. NCS Multistage Air Lock Tool at MD 4,200’ (5 ½”-17#-P-110)

BOP/ BOPE Guidelines

| Casing | Test Required | Rig Pump | Third Party | Test Pressure |
| --- | --- | --- | --- | --- |
| 9 5/8" | X |  | x | 3000# |
| 7”- 5 ½” |  |  |  |  |

The BOP/BOPE test shall include a low pressure test from 250 to 300psi.  The test will be held for a minimum of 10 minutes if test is done with a test plug and 30 minutes without a test plug.

All casing strings below the conductor shall be pressure tested to 0.22 psi per foot of casing string length or 1500 psi, whichever is greater, but not to exceed 70 percent of the minimum internal yield. Hold pressure for 30 minutes.

Mack Energy Corporation Drilling Guidelines

The Mack Energy Corporation representative on a job is responsible for notifying the NMOCD and BLM prior to Spud, Running Csg, Cementing, Testing Wellhead/BOP, and Testing the Csg.

Mack Energy Corporation will follow the BLM Onshore Oil and Gas Order No. 2 Drilling Operations guidelines as well as the NMOCD rules for drilling.

All pertinent information must be recorded on the daily reports and on the IADC Book.   This is to include all test information as well as who was contacted at the appropriate government agency.  This information needs to include a date and time.  Record all BOP Drills and Slow Pump Rate and Pressures. BOP Drills will be performed daily by each tour and Blind Rams will be function tested on trips.