2188A Ø6-3/4” 513-XR(K)(BR)

2188A was made for Callon to drill the wolfcamp lateral on a motor with a RSS. Current bits of choice are Baker DD405T, Ultera U613M, CF611, Securty MMD64DM. 2188 is based on the 2126. Changed to 13 mm cutters in the cone. More aggressive in cone and slightly more aggressive on the shoulder. Designed to drill faster to mitigate abrasive wear and thermal damage to cutters and erosion to the bit body.

Features:

- 4”Gage: 1” at gage, 3” 0.8° Taper

Bit Body:

Ø6-3/4”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

12.5° Cone

4” Gage

Open Face Volume = 9.995 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.102” | 1.294” | 0.852 |
| Center of round | 1.233” | 1.832” | 0.673 |
| Gauge | 1.140” | 1.986” | 0.574 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Back rakes:

Cone 		12.5°

Nose 		12.5°

Shoulder 	12.5-30°

Gage		35°

Components:

05 - 16/32 Nozzle Sleeves

05 - 55 Series Nozzles

34 - 1313 Primary Cutters

05 - 1111 Cutters

06 - 3/8” TCI Knuckles