2190A Ø6-1/8” 516-SR(K)(BR)

2190 was made to drill Lower Marmaton Laterals in the Anadarko Basin, Texas Panhandle for Henley & Associates. Carbonate formation just above the granite wash. Lateral, in at 7,015’, out at 11,900’. Bent Motor, .64 rev/gal (@275 gal/min  Total rpm = 240). Knuckles in cone (80’/hr @ 240rpm).

Features:

- 3”Gage: 1” at gage, 2” 0.3° Taper

Bit Body:

Ø6-1/8”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

12.5° Cone

3” Gage

Open Face Volume = 8.057 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.916” | 1.282” | 0.715 |
| Center of round | 1.200” | 1.447” | 0.829 |
| Gauge | 0.982” | 1.484” | 0.662 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Back rakes:

Cone 		17.5°

Nose 		20°

Shoulder 	20-30°

Gage		35°

Components:

05 - 16/32 Nozzle Sleeves

05 - 55 Series Nozzles

17 - 1613 Primary Cutters

10 - 1313 Cutters

03 - 3/8” TCI Knuckles

10 - 1/2” TCI Gage