1. Project Overview

- Lower Marmaton Laterals in the Anadarko Basin, Texas Panhandle. Carbonate formation just above the granite wash.

- Henley & Associates (Power Rig 2,4 & Texas American)

- Lateral, in at 7,015’, out at 11,900’

- Bent Motor, .64 rev/gal (@275 gal/min  Total rpm = 240)

2. Performance Target

- Current bits of choice:

Smith: MSi613, R616, R613, MDi516, MDi416

Baker: QT406FH, DPD405X, TD405XX, TD505X

- Current drilling performance:

High of 70’/hr., average around 30-40’/hr.

Of 7 wells, 1 was done with one bit, 3 with 2 bits, and 3 with 3 bits

- Historical CapRock performance (none)

- Target drilling performance

End goal is to change the interval to 1 or 2 bits, and maintain (if not 	improve) ROP.

Goal for first run is to get good data point to know what changes need to be

made to reach end goal.

3. Design Concepts under consideration

- 5 blades, 16mm

- Knuckles in cone (80’/hr @ 240rpm)

- 3” Gage; 1 @ gage, 2 @ .3°

- BR

- Backrake of 17.5° in cone, 20° on nose, 20-30° on shoulder, 35° Trimmers.

- 12.5° Cone angle (profile)

4. Field Test Plan

- 1 Bit to be built

- No ship date (Spec sheet first)