2192A Ø8-3/4” 616-SR

2192 is a vertical bit on a motor for Henley & Associates in North Texas/ Oklahoma Panhandle. In at 1,800’ to 5,700’. Total rpm of 200. Goal is drill 3900' in <40 hours. Aggressive back rake scheme with no back up elements. 3" gage (1.5"@gage and 1.5"@0.3°). Plan is to build one and deliver by 7/20.

Bit Body:

Ø8-3/4”

6 Blades

3 primary blades

3 secondary blades

65/55/60/60/65/55 Blade angles

15° Cone

3” Gage; 1.5” @ gage, 1.5” @ 0.3°

Open Face Volume =  in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.909” | 1.347” | 0.674 |
| Center of round | 1.568” | 1.646” | 0.953 |
| Gauge | 1.240” | 1.770” | 0.701 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-25°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

34 - 1613 Cutters