2193A Ø8-1/2” 813-SR+(K)

2193 is a deep vertical bit for SWD wells by XTO in eddy county. In at 10,900’ to 15,100’. Rotary at 55-65 rpm. Goal is 800’ to 1200’ per bit to save trips. 3 backup cutters on 4 blades trail by two blades and are set to engage at .04”/rev. Knuckles in cone set to engage at .05”/rev. Current belief is bit of choice has impact damage, so this design has a durable backrake for smoother drilling, 8 blades, and backups that will do some of the work. Quick redesign of 2191 after talking with Paul Pastusek

Bit Body:

Ø8-1/2”

7 Blades

3 primary blades

4 secondary blades

46/50/46/54/58/50/56 Blade angles

15° Cone

4” Gage; 1” @ gage, 3” @ 0.3°

Open Face Volume = 20.726 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.005” | 1.445” | 0.696 |
| Center of round | 1.350” | 1.652” | 0.817 |
| Gauge | 1.060” | 1.661” | 0.638 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		20°

Nose 		20°

Shoulder 	20-25°

Gage		35°

Components:

07 - 22/32 Nozzle Sleeves

07 - 60 Series Nozzles

45 - 1313 Cutters

11 - 1111 Cutters

03 - 1/2” PDC Knuckles

07 - 1/2” TCI Knuckles

14 - 3/8” TCI Knuckles (test as aid in hardmetal thickness for new builds, 2 per blade)