| Critical Limiter | Preferred Drill Bit Feature(s) | Original Design | New Design (07/06-2018) |
| --- | --- | --- | --- |
| Suppressing Bit Whirl and Stick Slip | Depth of Cut Elements - add1-2 klb WOB @ target DOC, show graph of engagement |  |  |
| Borehole Quality - Suppress Patterns | Equivalent to 0.3 to 0.6 degree relieved 3-4 Inch gauge, (For BUR < 12⁰/100ft) , No active cutters on pads |  |  |
| Improve Structural Integrity | Long substrate cutters 16x13 mm, 13mmx13mm, etc. 3 blades to center Avoid smaller cutters in cone |  |  |
| Suppress Gauge Trimmer Damage | Drill Out Feature Grind Flats on Trimmers |  |  |
| Aggressiveness / Durability | DOC Elements – see above |  |  |
|  | Back Rake- Cone 17-20⁰ Nose 20-25⁰ Shoulder 20-25⁰ Gauge Trimmers 30⁰+ Back Rake |  |  |
|  | 0.010-0.012 inch Cutter Chamfer Typical, 0.018 inch chamfer maximum |  |  |
| Select Bits for Cuttings Ribbon Flow | 2-3x Cutter Diameter Stand Off at Center No Pinch Points No buried cutters without room for chip flow |  |  |
| Minimize Balling | 2 nozzles per main blade on 12 ¼ and larger, 25-35% Junk slot Area Show CFD results: Ratio of %flow / %cuttings per blade Straight or minimum blade curvature No back up cutters in balling applications Polished cutter faces are preferred |  |  |
| Minimize Swabbing | 25-35% Junk Slot Area Minimum Gauge Pad Slant – clear line of sight down junk slots |  |  |
| Durability / Rock Strength | Blade Count – based on rock strength, interfacial severity, offsets Back up Cutters – come in by nose, recessed 0.020-0.050 inch Cutter Size 16mm+ |  |  |
| Select Cutters / Material based on application | Tangential Toughness, Axial Toughness, Wear, Thermal Damage – show data and reasoning for selection, Polished cutter faces are preferred |  |  |
| Stability vs Life | Medium Parabolic Bit Profile |  |  |
| Blade Strength | Finite Element Analysis (FEA) of blade strength relative to API pin connection |  |  |
| Erosion vs Life | Matrix vs Steel - usually both will work, evaluate per application |  |  |
| Document the Limiters & Track Design Changes | Commit to a forensic analysis of every run, with proposed design changes and reasons for and risks of the changes |  |  |