2195A Ø8-3/4” 613-SR+(K)(BR)

2195 was designed for Callon, Howard County, Wolfcamp A wells. Made to drill the curve and lateral until the bit slows down. 10°/100’. ½” TCI in gage to help prevent gage wear. 15° cone fades to a 25° shoulder (aggressive due to competitors bit photo on yammer and going from 16mm cutters to 13mm cutters densely spaced). 35° Trimmers. Knuckles in cone, 2 backups on outer shoulder. Step profile to increase side cutting ability. 0.075”/rev for Knuckle placement based on an analysis of Callon Wolfcamp curve bit DOC’s.

Bit Body:

Ø8-3/4”

6 Blades

3 primary blades

3 secondary blades

60/60/58/62/55/65 Blade angles

5° Cone

2” Gage, 1”@ Gage, 1”@ 0.3°

Open Face Volume = 16.622 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.929 | 1.453 | 0.639 |
| Center of round | 1.354 | 1.700 | 0.796 |
| Gauge | 1.305 | 1.760 | 0.741 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	16-25°

Gage		30-35°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

64 - 1313 Cutters

06 - 1/2" PDC Bullets