| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Secondary) | 5/8”, 1/2” |  |
| Cutter Count (Total, Gage, Back Reaming) | 51, 10, 5 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 2.00 |  |
| Nozzle Count (Primary, Sec.) | 5 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 17.57 |  |
| Max. TFA (Sq. Inches) | 1.86 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 16,488 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 14.22 |  |
| Make Up Length (Inches) | 10.10 |  |
| Bit Breaker | SKB-21010 |  |
| Connection | Ø4-1/2" API Reg. |  |
| Approx. Weight (Lbs.) | 98 |  |

Standard Features:

Premium grade cutters maximize ROP and Life

Nozzle configuration and large open face volume optimize blade cleaning and cuttings removal

516-SR+(K)(BR) Nomenclature Explained:

| - • | - 5 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - SR+ | - = | - Partial Backup Row of Cutters |
| - • | - K | - = | - Bullets |
| - • | - BR | - = | - Back Reamers |