1. Project Overview

- Fasken

- LS wells (Sprayberry)

- 8-3/4” in at 8600’

- motor, curve

- 154 to 174 rpm

2. Performance Target

- Security, MM55DM

- Target drilling performance:

- Smooth drilling, toolface control

- ≈38’/hr

(0.049”/rev)

3. Design Concepts under consideration

- 5° cone, 15° backrake in cone

- 15°-25° shoulder

- 2” Gage, 1”@ gage, 1”@ 0.3°

- .030”/rev knuckle placement, trailing by two primary blades

- two backup cutters on outer shoulder

4. Field Test Plan

- Spec sheet to Jim ASAP for approval from Cory Frederick

- Build 1

- Target shipping date: August 30, 2018