2198A Ø8-3/4” 616-SR(K)(BR)

2198 was designed for Callon, Casselman wells. The Casselman lease has been the most difficult curve area with multiple bit styles, bends on motors, etc. Engineering and sales discussed a 5° cone angle, 13mm cutters in the cone to the nose, and 16mm cutters on the shoulder to help the bit go sideways. Knuckles in cone. Step profile to increase side cutting ability. Also has a more aggressive backrake scheme compared to 2195.

Bit Body:

Ø8-3/4”

6 Blades

3 primary blades

3 secondary blades

60/60/58/62/55/65 Blade angles

5° Cone

3” Gage, 1”@ Gage, 2”@ 0.8°

Open Face Volume = 16.622 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.929 | 1.453 | 0.639 |
| Center of round | 1.354 | 1.700 | 0.796 |
| Gauge | 1.305 | 1.760 | 0.741 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	16-20°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

26 - 1613 Cutters

22 - 1313 Cutters

06 - 1/2" TCI Bullets

30 - 1/2" TCI Gage