| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage) | 100,12 |  |
| Blade Count | 6 |  |
| Gage Length (Inches) | 7.00 |  |
| Nozzle Count (Primary, Sec.) | 9 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 83.93 |  |
| Max. TFA (Sq. Inches) | 3.34 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 57,735 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 28.48 |  |
| Make Up Length (Inches) | 23.36 |  |
| Bit Breaker | SKB-21047 |  |
| Connection | Ø7-5/8" API Reg. |  |
| Approx. Weight (Lbs.) | 650 |  |

Standard Features:

Large open face volume assists cuttings removal

Nine nozzles optimize blade cleaning and cuttings removal

Premium grade cutters maximize ROP and Life

616-SR+9(K) Nomenclature Explained:

| - • | - 6 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - SR+ | - = | - Partial Backup Cutters |
| - • | - 9 | - = | - Number of Nozzles |
| - • | - K | - = | - Bullets |

- *Interchangeable nozzles.