2206 Ø12-1/4” 616-XB12(K)(BR)

2206 Uses the backrake scheme like the 2177. Normal profile (non-ellipse). Going after the 2138 interval with the intent to increase the TD rate on Fasken H&P 497. We currently are tripped 500 to 1000 ft before TD. Amanda Ohm’s Idea: Place the backup cutters on their own partial blade (Double Blade) that extends from the primary blades, with its own junk slot. (3 Primary blades only, secondaries are normal.)

Features:

- 6 blades, with 3 “double blades” to increase durability.

Bit Body:

Ø12-1/4”

6 Blades

3 primary blades

3 secondary blades

50/70/50/70/55/65 Blade angles (All double blades are at 30°)

15° Cone

6” Gage

Open Face Volume = 42.052 in^3

Height to Width Ratio:

| (Primary/Secondary) | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.264” / 1.264” | 1.273” / 1.625” | 0.993 / 0.779 |
| Center of round | 1.583” / 1.583” | 1.555” / 2.261” | 1.018 / 0.700 |
| Gauge | 1.680” / 1.680” | 1.669” / 2.491” | 1.007 / 0.674 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Backup elements are under exposed @ .333”/rev.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

09 - 22/32 Nozzle Sleeves

03 - 16/32 Nozzle Sleeves

09 - 60 Series Nozzles

03 - 45 Series Nozzles

73 - 1613 Primary Cutters

21 - 1313 Cutters

09 - ½” TCI Knuckles