2207A Ø7-7/8” 716-SR+(K)

Blackbeard wanted a 7-bladed 7-7/8" to drill from 600-6500' in one bit consistently. We designed 2197 which is a 6-bladed bit and Blackbeard ran it one time but made it to TD. Since then Ulterra designed a 7-bladed that has consistely made it to TD. Took the 2176 and made it a 7-blade design.

Features:

- Bullets and backups are set to engage at .40”/rev.

Bit Body:

Ø7-7/8”

7 Blades

3 primary blades

4 secondary blades

52/50/54/46/56/48/54 Blade angles

15° Cone

4” Gage; Straight

Open Face Volume =  in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. |  |  |  |
| Center of round |  |  |  |
| Gauge |  |  |  |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

07 - 22/32 Nozzle Sleeves

07 - 60 Series Nozzles

38 - 1613 Cutters

6 - 1313 Cutters

18 - 1/2” TCI Bullets