2208A Ø12-1/4” 716-XR10(K)(BR)

2208 is a durable 716, intended to be the second intermediate bit, following 2209.Eddy and Lea counties, with XTO. 7,500’ to 10,500’, vertical on motor (.16 rev/gal). Backups are set at 50’/hr @ 180 rpm (.056”/rev). Target is to be between 50’/hr and 100’/hr.

Features:

- Bullets in cone to nose for ROP limiters and impact arrestors.

- One carbide bullets on gage to absorb lateral impacts.

- Seven blades to get more diamonds on the shoulder.

Bit Body:

Ø12-1/4”

7 Blades

3 primary blades

4 secondary blades

46/48/48/54/58/50/56 Blade angles

15° Cone

6” Gage

Open Face Volume = 43.059 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.264” | 1.688” | 0.749 |
| Center of round | 1.583” | 2.318” | 0.683 |
| Gauge | 1.680” | 2.547” | 0.660 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Knuckles are under exposed in the cone and nose at .015”; all other backup elements are set at .056”/rev.

Back rakes:

Cone 		17.5°

Nose 		17.5°

Shoulder 	17.5-30°

Gage		40°

Components:

10 - 22/32 Nozzle Sleeves

10 - 60 Series Nozzles

65 - 1613 Primary Cutters

42 - 1313 Cutters

17 - ½” TCI Knuckles