| Critical Limiter | Preferred Drill Bit Feature(s) | New Design (07/06-2018) |
| --- | --- | --- |
| Suppressing Bit Whirl and Stick Slip | Depth of Cut Elements - add1-2 klb WOB @ target DOC, show graph of engagement | Yes |
| Borehole Quality - Suppress Patterns | Equivalent to 0.3 to 0.6 degree relieved 3-4 Inch gauge, (For BUR < 12⁰/100ft) , No active cutters on pads | Yes None |
| Improve Structural Integrity | Long substrate cutters 16x13 mm, 13mmx13mm, etc. 3 blades to center Avoid smaller cutters in cone | Yes Yes Yes |
| Suppress Gauge Trimmer Damage | Drill Out Feature Grind Flats on Trimmers | Yes Yes |
| Aggressiveness / Durability | DOC Elements – see above | Yes |
|  | Back Rake- Cone 17-20⁰ Nose 20-25⁰ Shoulder 20-25⁰ Gauge Trimmers 30⁰+ Back Rake | Design for aggressiveness Design for aggressiveness Design for aggressiveness Yes |
|  | 0.010-0.012 inch Cutter Chamfer Typical, 0.018 inch chamfer maximum | Yes - .016” |
| Select Bits for Cuttings Ribbon Flow | 2-3x Cutter Diameter Stand Off at Center No Pinch Points No buried cutters without room for chip flow | Yes Yes Yes |
| Minimize Balling | 2 nozzles per main blade on 12 ¼ and larger, 25-35% Junk slot Area Show CFD results: Ratio of %flow / %cuttings per blade Straight or minimum blade curvature No back up cutters in balling applications Polished cutter faces are preferred | 23% Not Yet Yes No Balling expected Not Yet |
| Minimize Swabbing | 25-35% Junk Slot Area Minimum Gauge Pad Slant – clear line of sight down junk slots | 23% Yes |
| Durability / Rock Strength | Blade Count – based on rock strength, interfacial severity, offsets Back up Cutters – come in by nose, recessed 0.020-0.050 inch Cutter Size 16mm+ | 7 Blades Back up cutters are set at DOC target Yes |
| Select Cutters / Material based on application | Tangential Toughness, Axial Toughness, Wear, Thermal Damage – show data and reasoning for selection, Polished cutter faces are preferred |  |
| Stability vs Life | Medium Parabolic Bit Profile | Yes |
| Blade Strength | Finite Element Analysis (FEA) of blade strength relative to API pin connection | Steel Blades |
| Erosion vs Life | Matrix vs Steel - usually both will work, evaluate per application | No heavy erosion expected |
| Document the Limiters & Track Design Changes | Commit to a forensic analysis of every run, with proposed design changes and reasons for and risks of the changes | If enough data is provided to support design change decisions, Yes. |