2218A Ø12-1/4” 716-XR10(K)(BR)

2218 is an updated version of 2210. The profile has more cutter exposure on the shoulder. The backup cutters have been slightly moved. 2210 is for Callon, Howard county; 1,100’ to 6,800’, vertical on motor (.16 rev/gal). Target is 120’/hr overall and to TD in one bit.

Bit Body:

Ø12-1/4”

7 Blades

3 primary blades

4 secondary blades

49/47/46/54/58/50/56 Blade angles

15° Cone

6” Gage

Open Face Volume = 46.549 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.264” | 1.688” | 0.749 |
| Center of round | 1.583” | 2.318” | 0.683 |
| Gauge | 1.680” | 2.547” | 0.660 |

Primary Cutting Structure:

Non-redundant; F-7/3 Reverse Spiral; Secondary blades are flush to primary; Knuckles are under exposed in the cone and nose at .025”;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

10 - 22/32 Nozzle Sleeves

10 - 60 Series Nozzles

63 - 1613 Primary Cutters

41 - 1313 Cutters

17 - ½” TCI Knuckles