2220A Ø8-3/4” 716-XR(K)

2220 was designed for EOG, Lea county, NM. Customer wants 7 blade 8-3/4 bit for two-bit interval down to 9,600’. Vertical well, planned pull point around 6,000’. Target is 180’/hr over all, with 180rpm. There is no offset data that goes this deep with a 8-3/4 bit. This first design will be a data point to let us know which direction to go from here.

Bit Body:

Ø8-3/4”

7 Blades

3 primary blades

4 secondary blades

49/47/46/54/58/50/56 Blade angles

15° Cone

3.2” Gage

Open Face Volume = 21.444 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.138 | 1.386 | 0.821 |
| Center of round | 1.345 | 1.732 | 0.777 |
| Gauge | 1.177 | 1.822 | 0.646 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		30°

Components:

07 - 22/32 Nozzle Sleeves

07 - 60 Series Nozzles

45 - 1613 Cutters

19 - 1313 Cutters

07 - 1/2" TCI Bullets

06 - 3/8" TCI Bullets