2225A Ø6-1/8” 416-SR(K)(BR)

2225 is a 4 blade version of 2190, which is for lateral wells drilled by Quanah. In at 7,015’, out at 13,000’. Bent Motor. Knuckles on nose and shoulder (200’/hr @ 180rpm).

Features:

- 3”Gage

Bit Body:

Ø6-1/8”

4 Blades

2 primary blades

2 secondary blades

85/95/85/95 Blade angles

12.5° Cone

3” Gage

Open Face Volume = 9.359 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.916” | 1.066” | 0.859 |
| Center of round | 1.199” | 2.057” | 0.583 |
| Gauge | 0.982” | 2.367” | 0.415 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Back rakes:

Cone 		17.5°

Nose 		20°

Shoulder 	20-30°

Gage		35°

Components:

04 - 22/32 Nozzle Sleeves

04 - 60 Series Nozzles

17 - 1613 Primary Cutters

04 - 1313 Cutters

08 - 1/2” TCI Knuckles

12 - 1/2” TCI Gage