2227A Ø6-3/4” 513-XR(K)(BR)

2227A is the first Matrix lateral bit. Based on the 2188 and pictures on Yammer of a security MMD/GT54.

Features:

- 4” Gage: 1” at gage, 3” 0.8° Taper

Bit Body:

Ø6-3/4”

5 Blades

3 primary blades

2 secondary blades

76/64/80/64/76 Blade angles

12.5° Cone

4” Gage

Open Face Volume = 9.082 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.881” | 1.344” | 0.656 |
| Center of round | 1.195” | 1.833” | 0.652 |
| Gauge | 1.273” | 2.063” | 0.617 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Back rakes:

Cone 		12.5°

Nose 		12.5°

Shoulder 	12.5-30°

Gage		35°

Components:

05 - 55 Series Nozzles

36 - 1313 Primary Cutters

15 - 1111 Cutters

03 - 3/8” TCI Knuckles