2231A Ø8-1/2” 616M-XR(BR)

2231 cutter set is based on the 2183 made more aggressive. The bit is the second matrix bit. Designed for the same application as the 2184 hybrid bit.

Bit Body:

Ø8-1/2”

6 Blades

3 primary blades

3 secondary blades

59/62/59/59/62/59 Blade angles

10° Cone

4” Gage

Open Face Volume = 19.234 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.876” | 1.252” | 0.700 |
| Center of round | 1.310” | 1.992” | 0.658 |
| Gauge | 1.269” | 2.233” | 0.568 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		12.5°

Nose 		12.5°

Shoulder 	12.5-25°

Gage		30°

Components:

06 - 60 Series Nozzles

35 - 1613 Cutters

22 - 1313 Cutters