2232 Ø12-1/4” 616-XB12(K)(BR)

2232 is a redesign of 2213. 1- Profile is different (elliptical) to get more cutters on the shoulder. 2- Cutter layout is now a 3 pattern to 6 to 9 (was 6 pattern with double blades acting like backup cutters). Changes are to make the bit more durable.

Features:

- 6 (9) blades, with 3 “double blades” to increase durability.

Bit Body:

Ø12-1/4”

9 Blades

3 primary blades

6 secondary blades

50/40/30/50/40/30/50/40/30 Blade angles

15° Cone

5” Gage

Open Face Volume = 33.901 in^3

Height to Width Ratio:

| (Primary/Secondary) | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.179” / 1.179” | 1.273” / 1.625” | 0.926 / 0.726 |
| Center of round | 1.322” / 1.322” | 1.507” / 2.161” | 0.877 / 0.612 |
| Gauge | 1.680” / 1.680” | 1.669” / 2.491” | 1.007 / 0.674 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Backup elements are under exposed @ .1”/rev. (knuckles are at .15”/rev)

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-25°

Gage		30°

Components:

09 - 22/32 Nozzle Sleeves

03 - 16/32 Nozzle Sleeves

09 - 60 Series Nozzles

03 - 45 Series Nozzles

81 - 1613 Primary Cutters

23 - 1313 Cutters

09 - ½” TCI Knuckles