2235A Ø12-1/4” 716-XR10(K)(BR)

2235 is to get 12-1/4” business back. Going after 2138 on Fasken. Backups are set at 80’/hr @ 180 rpm (.089”/rev). This design uses a more traditional CapRock profile with cutter spacing that is more aggressive than a typical 7 blade bit, but with a dense cone. The backrake in the cone is set durable with an aggressive shoulder, then a durable gage. Idea is to carry the weight in the cone, not the shoulder.

Bit Body:

Ø12-1/4”

7 Blades

3 primary blades

4 secondary blades

51/49/48/54/54/50/54 Blade angles

15° Cone

6” Gage

Open Face Volume = 41.767 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.208” | 1.622” | 0.745 |
| Center of round | 1.551” | 2.235” | 0.694 |
| Gauge | 1.692” | 2.491” | 0.679 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; backup elements are set at .089”/rev.

Back rakes:

Cone 		22°

Nose 		22-15°

Shoulder 	15-37°

Gage		40°

Components:

10 - 22/32 Nozzle Sleeves

10 - 60 Series Nozzles

66 - 1613 Primary Cutters

42 - 1313 Cutters

07 - ½” TCI Knuckles