| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage, Back Ream) | 87, 9, 9 |  |
| Blade Count | 6 |  |
| Gage Length (Inches) | 4.00 |  |
| Nozzle Count (Primary, Sec.) | 6, 3 |  |
| Nozzle Series (Primary, Sec.) | 60, 45 |  |
| JSA (Sq. Inches) | 15.437 |  |
| Max. TFA (Sq. Inches) | 2.82 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 40,563 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | *18.36 |  |
| Make Up Length (Inches) | *13.48 |  |
| Bit Breaker | SKB-21011 |  |
| Connection | Ø6-5/8" API Reg. |  |
| *Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs. to approximate weight). Approx. Weight (Lbs.) | *190 |  |

Taylor-Made Features:

Premium grade cutters maximize ROP and Life

Selectively positioned bullets for smooth drilling

Nozzle configuration and large open face volume optimize blade cleaning and cuttings removal

Double blade design extends bit life for increased footage

616-XB9(K)(BR) Nomenclature Explained:

| - • | - 6 | - = | - Number of Blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - XB | - = | - Backup Cutters & Double Blade |
| - • | - 9 | - = | - Number of Nozzles |
| - • | - K | - = | - Bullets |
| - • | - BR | - = | - Back Reaming Cutters |