2236 Ø9-7/8” 616-XB9(K)(BR)

2239 is a scaled down dragon design for Diamondback. Based on 2232.

Features:

- 6 (9) blades, with 3 “double blades” to increase durability.

Bit Body:

Ø9-7/8”

9 Blades

3 primary blades

6 secondary blades

50/40/30/50/40/30/50/40/30 Blade angles

15° Cone

4” Gage

Open Face Volume = 19.115 in^3

Height to Width Ratio:

| (Primary/Secondary) | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.042” / 1.042” | 0.959” / 1.547” | 1.087 / 0.674 |
| Center of round | 1.137” / 1.137” | 1.164” / 1.850” | 0.977 / 0.615 |
| Gauge | 1.242” / 1.242” | 1.282” / 1.996” | 0.969 / 0.622 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Backup elements are under exposed @ .1”/rev. (knuckles are at .179”/rev)

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-25°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

03 - 16/32 Nozzle Sleeves

06 - 60 Series Nozzles

03 - 45 Series Nozzles

64 - 1613 Primary Cutters

23 - 1313 Cutters

06 - ½” TCI Knuckles