2240A Ø7-7/8” 416-XB8

2240 is a 4-blade dragon bit. The 4-blade configuration allows more cutters in the cone than on a traditional 6 blade bit. It also is more durable on the shoulder due to two extra blades. Should be slower than 2176 and 2197. Three start pattern, then 4… to eight pattern.

Features:

- Double blades on all 4 blades

Bit Body:

Ø7-7/8”

4 Blades

3 primary blades

1 secondary blades

90/87/90/93 Blade angles (50/40/47/40/50/40/53/40)

15° Cone

4” Gage

Open Face Volume = 14.309 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.987” | 1.124” | 0.878 |
| Center of round | 1.064” | 1.220” | 0.872 |
| Gauge | 0.861” | 1.241” | 0.694 |

(Height/width ratios look like week blades, but on the double blades the inner profile is raised up, so the effective ratios are not as week.)

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

04 - 22/32 Nozzle Sleeves

04 - 16/32 Nozzle Sleeves

04 - 60 Series Nozzles

04 - 45 Series Nozzles

43 - 1613 Cutters