| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 3/64” |  |
| Cutter Count (Total, Gage) | 26,6 |  |
| Blade Count | 6 |  |
| Gage Length (Inches) | 2.50 |  |
| Nozzle Count (Primary, Sec.) | 6 |  |
| Nozzle Series (Primary, Sec.) | 35 |  |
| JSA (Sq. Inches) | 4.924 |  |
| Max. TFA (Sq. Inches) | 0.66 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 15 |  |
| Make-up Torque (ft/lbs) | 4,613 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 9.69 |  |
| Make Up Length (Inches) | 6.31 |  |
| Bit Breaker | SKB-21013 |  |
| Connection | Ø2-7/8" API Reg. |  |
| Approx. Weight (Lbs.) | *20 |  |

*Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs. to approximate weight).

*Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs. to approximate weight).

Standard Features:

Large open face volume assists cuttings removal

Six nozzles optimize blade cleaning and cuttings removal

Premium grade cutters maximize ROP and Life

611-SR Nomenclature Explained:

| - • | - 6 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 11 | - = | - Primary Cutter Size (11mm) |
| - • | - SR | - = | - Single Row of Cutters |
| - • | - 6 | - = | - Number of Nozzles |

*Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs. to approximate weight).