2252 Ø10-5/8” 616-SR(K)

2252 is for XTO on the Dark Star Unit leases. Depth in around 1,200’ and depth out around 5,600’. Designed to average over 200’/hr over the whole interval. Undercut inner profile to increase JSA.

Features:

- 6 blades.

- F63 and 6-3 factor of 0.030”

Bit Body:

Ø10-5/8”

6 Blades

3 primary blades

3 secondary blades

58/62/58/62/58/62 Blade angles

15° Cone

5” Gage, 2.5” @ gauge, 2.5” @ 0.5°

Open Face Volume = 36.652 in^3

Height to Width Ratio:

| (Primary/Secondary) | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.179” | 1.380” | 0.854 |
| Center of round | 1.605” | 1.799” | 0.892 |
| Gauge | 1.676” | 1.919” | 0.873 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral F63; Secondary blades have 6-3 factor of 0.03”;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

49 - 1613 Primary Cutters

06 - ½” TCI Knuckles