New Design Request and Priority Setting

PDC Drill Bits

This procedure controls the selection, specification, and prioritization of engineering and manufacturing efforts to design new or improved PDC drill bits. It guides the evolutionary development of new and improved drill bits as needs are recognized and requested by the Sales force. Bit development activity originating elsewhere is outside the scope of this procedure. These other activities include cost and lead time reduction opportunities and engineering-driven product innovation.

The purpose of the design request form is to define a performance target to be achieved within a specific drilling application. Both the performance target and the drilling application must be clearly defined so that the bit design is targeted to meet a clear and specific need. This applies to new designs and design modifications requested by the Sales force.

Anyone in the sales organization may initiate a Design Request based on their recognition of revenue growth or defense, or other justifiable business benefit. Typically, the design request is completed by a sales manager or office or field sales personnel. The procedure is as follows:

1. Once the opportunity is recognized the initiator will complete a Design Request Form using their best effort to fill in all requested data on the form. The Design Request Form is given at the end of this document.

2. The initiator submits the Design Request to the US Sales manager for his review. Once approved, the US Sales Manager submits the request for action via M1.

3. The Engineering Manager assigns the Design Request to a product engineer via M1. The product engineer reviews it for technical feasibility, working with the initiator where change or clarification is needed. The Engineering Manager completes validation in M1.

4. Once validated, Leadership schedules the consumption of engineering and manufacturing resources. Resource constraints and short-term deadlines may require priority-setting among multiple engineering and manufacturing activities. In such cases, Leadership will review resource trade-offs to establish optimum priority among competing projects. The result will be a forecasted schedule of completion for the project stack, published by Operations via Bit Tracker email.

5. Product Design is executed via procedure 1002 Product Development Process. The Engineering Manager sends notification of design completion via M1.

6. Manufacturing Engineering completes NC programming to enable bit manufacture. The Engineering Manager sends notification of CAM programming via M1.

PDC Bit Design Request

Name:  Butch Bawcom

Date:   03/06/2025

Bit Type_______________________________________________________________

Bit Diameter:  Ø9.625

Connection Size and Type:   4 1/2” Reg

Number of Blades recommendation:   6

Primary Cutter Size recommendation:  16

Single or Double Row recommendation:  Double Row

Value Proposition____________________________________________________________

Complete this Excel sheet to evaluate the profitability of the Bit Design Request.

Only Design Requests that meet the profitability target will be prioritized for action.

The second file is a Drilling Cost Scenario Calculator to assist with your evaluation.

Requested Build Qty:   1

Application Details____________________________________________________________

CapRock Application Code:

If no application code, then explain the interval in detail:

Depth In:    4000ft

Depth Out:   7100 ft

Wellbore Trajectory (Vertical, Tangent, Curve, Lateral):  Vertical

If Curve, degrees per hundred build rate:   no

Application Details_(continued)___________________________________________________

Expected footage drilled under each of these conditions:

|  |  | Compressive Strength (unconfined) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Low <10 ksi | Medium 15 to 25 ksi | High >30 ksi |
| Formation | Shale | In: 1500 Out: 3000 | In: Out: | In: Out: |
|  | Sand | In: Out: | In: 3000 Out: 4600 | In: Out: |
|  | Lime | In: Out: | In: 4600 Out: 7600 | In: 7600 Out: 11600 |
|  | Other (Specify) | In: 11600 Out: 20000 | In: Out: | In: Out: |

Which of these intervals are problem formations causing bit damage, balling, or other problems?

Transitions from San Andres, Cherry Canyon,Brushy Canyon

Quantify expected RPM

(If ranges are given, averages will be used to formulate design. Total RPM will be the sum of these.)

Conventional Rotary:   60-70

Motor:  140-150

Rotary Steerable:  No

Performance Targets

What performance  targets are required to meet the profitability?  Quantify footage, ROP, and repair intervals required to achieve  sustained and profitable sales:

Target Footage for the interval: 7100 ft

Is the bit required to TD the hole section?  Yes

Target ROP for the full interval :  150 - 170 ft/hr

Maximum sustained ROP during a portion of the interval:    215 ft/hr

Number of runs between repairs: 1

OTHER SUPPORTING INFORMATION

Which customers will be using the bit? EnXL

In what Sales District will the bit be used?  WTNM

What are the current bits of choice? CR516

Are spec sheets and/or bit photos available? Yes

What are the typical dull condition and failure mechanisms of these bits of choice?  1-1-CT-SG-X-I- WT-TD

What assembly number will be assigned?

Sales and Customer Preferences

Provide a number or description to specify preferences for any of these characteristics:

|  | Salesman Preference | Customer Preference |
| --- | --- | --- |
| Leached Cutters (Yes or No) | Yes - Deep Leach | Yes - Deep Leach |
| Bit Length | TBD Engineering | Steerable for Tangent applications |
| Gauge Length | 4.75 |  |
| Number of Blades to Center | 3 |  |
| Number of Nozzles | 9 | 9 |
| Maximum TFA | 2.237in3 | 1.15 is popular |
| Junk Slot Area | TBD Engineering |  |
| Open Face Volume | TBD Engineering |  |
| Knuckles? (Yes or No) | Yes- Set Very low |  |
| Any Other Features | Reverse Spiral 43% Exposure |  |