New Design: 3934 is a redesign of 2268 (K3).

The 2268 (K3) is an aggressive design. It’s fast but does not reliably make TD. The failure mechanism of the 2268 (K3) is the shear strength of the PDC cutters located on the nose and taper of the primary blades. The focus of the redesign is to minimize the shear forces exerted on the primary cutters without taking away from the aggressiveness of the original design. We added steel support to the base of each PDC.  We have adjusted the direction of the PDC force vectors to decrease torque on the primary cutters (the force responsible for shearing the primary cutters of the 2268 (K3)).

The bit has all the same features and will have equivalent or better overall ROP with a higher TD rate.