Highlights of the Ø9.875” 616-XR(K)(BR) design:

- Elliptical profile = increased the diamond density on taper and shoulder.

- Shaped cutters are employed to improve cutting efficiency.

- The faces of the blades match the shape of the axe cutters to minimize washout between cutters.

- Nozzles are placed directly in front of planar PDC’s to minimize washout between cutters.

- Top-Load back-up cutters increase PDC shear loads (more support behind the primary cutting structure).

- Secondary blade webbing increases the rigidity of the secondary blades.

- The large radius at base of blades increases rigidity of the blades.

- The inner profile of bit body is smaller than the tool joint OD = Increased JSA.

- Blade relief in cone of bit allows for increased ROP.

- The back-up cutters on this bit will not engage until the primary cutters show significant signs of failure.

| Blade 1 | Blade 2 | Blade 3 | Blade 4 | Blade 5 | Blade 6 |
| --- | --- | --- | --- | --- | --- |

Radial Position: Position #1 is at the center of the bit. The position #’s increase with the radius.

Element Name:

- The # before the decimal is the blade # (the colors correspond to the drawings above).

- If the first # after the decimal is a 1, it’s a primary element position. If it is a 2, it’s a back-up position.

- The last two digits are the radial position.