NDR --- New Design Request & Priority Setting

This procedure controls the selection, specification, and prioritization of engineering and manufacturing efforts to design new or improved PDC drill bits. It guides the evolutionary development of new and improved drill bits as needs are recognized and requested by the Sales force. Bit development activity originating elsewhere is outside the scope of this procedure. These other activities include cost and lead time reduction opportunities and engineering-driven product innovation.

The purpose of the design request form is to define a performance target to be achieved within a specific drilling application. Both the performance target and the drilling application must be clearly defined so that the bit design is targeted to meet a clear and specific need. This applies to new designs and design modifications requested by the Sales force.

Anyone in the sales organization may initiate a Design Request based on their recognition of revenue growth or defense, or other justifiable business benefit. Typically, the design request is completed by a sales manager or office or field sales personnel. The procedure is as follows:

- Once the opportunity is recognized the initiator will complete a Design Request Form using their best effort to fill in all requested data on the form. The Design Request Form is given at the end of this document.

- The initiator submits the Design Request to the US Sales manager for his review. Once approved, the US Sales Manager submits the request for action via M1.

- The Engineering Manager assigns the Design Request to a product engineer via M1. The product engineer reviews it for technical feasibility, working with the initiator where change or clarification is needed. The Engineering Manager completes validation in M1.

- Once validated, Leadership schedules the consumption of engineering and manufacturing resources. Resource constraints and short-term deadlines may require priority-setting among multiple engineering and manufacturing activities. In such cases, Leadership will review resource trade-offs to establish optimum priority among competing projects. The result will be a forecasted schedule of completion for the project stack, published by Operations via Bit Tracker email.

- Product Design is executed via procedure 1002 Product Development Process. The Engineering Manager sends notification of design completion via M1.

- Manufacturing Engineering completes NC programming to enable bit manufacture. The Engineering Manager sends notification of CAM programming via M1.

NDR --- New Design Request

Bit Type

Bit Diameter:   Ø12.25”

Connection Size and Type:   6-5/8” reg

Number of Blades recommendation:   6

Primary Cutter Size recommendation:   16

Single or Double Row recommendation:   SR+ or XR

Three Ideal Prospects

Prospect 1:   Discovery-Fasken

Prospect 2:   SEM Operating- Endeavor/Diamondback

Prospect 3:   Apache

Value Proposition

Complete this Excel sheet to evaluate the profitability of the Bit Design Request.

Only Design Requests that meet the profitability target will be prioritized for action.

The second file is a Drilling Cost Scenario Calculator to assist with your evaluation.

Requested Build Qty:   2

Performance Targets (required to retain profitable business)

Target Footage for the interval:  6500-7500’

Is the bit required to TD?:   Yes

Target ROP for the full interval:  225 ft/hr

Maximum instantaneous ROP:   450 ft/hr

Maximum sustained ROP:   300 ft/hr

Application Details

Wellbore Trajectory (Vertical, Tangent, Curve, Lateral, VCL):  Vertical and Tangents to 8-10 degrees in Intermediates

If Curve, degrees per hundred build rate:   n/a

Rotary rpm:   60-80

Motor/RSS rpm:   120-165

Depth In:   1500

Depth Out:  8000-9000’

Expected footage drilled under each of these conditions:

|  |  | Compressive Strength (unconfined) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Low <10 ksi | Medium 15 to 25 ksi | High >30 ksi |
| Formation | Shale | In: 1500’ Out: 3100’ | In: x Out: x | In: x Out: x |
|  | Any-Sand | In: x Out: | In: 3100’ Out: 4200’ | In: x Out: x |
|  | Lime | In: x Out: | In: 4200’ Out: 5500’ | In: 5500’ Out: 9000’ |
|  | Other (Specify) | In: x Out: x | In: x Out: x | In: x Out: x |

Which of these intervals are problem formations causing bit damage, balling, or other problems?

3750’-hard transition,  Grayburg Cap around 4200’ transition hard

Other Supporting Information

Which customers will be using the bit?  Fasken – SEM – Discovery – Apache - Endeavor/Diamondback

In what Sales District will the bit be used?  WTNM

What are the current bits of choice?   TKF66-GT65-WAV616-RPS616

Are spec sheets and/or bit photos available?   Not at present

What are the typical dull condition and failure mechanisms of these bits of choice?  2-3 CT-SG

Sales and Customer Preferences

|  | Salesman Preference | Customer Preference |
| --- | --- | --- |
| PDC Type (Leached?) | Deep | Deep |
| Bit Make-up Length | 15.68 | 15.68 |
| Gauge Length | 5.9 | 5-6 |
| Number of Blades to Center | 3 | 3 |
| Number of Nozzles | 9 | 9 |
| Maximum TFA | 3.34 | 3.34 |
| Knuckles? (Yes or No) | Yes | Yes |
| Any Other Features | 43% exposure 20 degree plus cone angle xxx | xxx xxx xxx |

Additional Notes for Engineering:

xxx