General Characterization of Design Layouts and Ripper Configurations

Basic Layout Types:

- Redundant (14-25-36; 14-36-25)

  - Cutlet forces: push perpendicular to the bit profile.

  - Application: General purpose durable layout

- Forward Spiral (123456)

  - Cutlet forces: predominantly radial; compress the bit body (requires more torque)

  - Application: Surface bits and smaller rigs with limited WOB that want higher ROP; will require more torque.

- Reverse Spiral (165432)

  - Cutlet forces: predominantly axial (requires more WOB)

  - Application: Curve and laterals for higher overall ROP because of increased tool face control.

Additional Layouts:

- 6-3 (can be applied to any bit)

  - Makes any design more aggressive, but at the cost of being more vulnerable at low in/rev (effectively a three bladed bit).

- F-Type (4 variants)

  - Similar cutlets and aggressiveness of a 6-3, but all blades are equally exposed (more durable than a 6-3 at low in/rev)

  - A 6-blade F-Type bit is approximately equivalent to running a 5-bladed bit.

- F-type where Sec. blade patterns have a radial shift (infinite variants).

  - These designs are employed to make an F-Type bit more durable.

- Dragon (where trailing back-up cutters have their own blades)

  - These back-up positions do not pack off and slow ROP.

- Mixed Cutlet Type (one variant currently known)

  - This design employs all three cutlet shapes in one layout (Forward, Reverse, Redundant).

Cutter Configurations:

- C0 = Full Round PDC’s

- C1: Ripper cutters on secondary blades (No rippers in the cone)

- C2: Alternate Ripper cutters on primary blades

Basic Layout Types and Ripper Configurations:

Note: all three of these designs would be more aggressive than a bit with full round cutters in every position.

Note: depending on the design of the bit C1 or C2 could have more rippers in the layout. It would need to reviewed on a case-by-case basis.

Additional Layout Types and Ripper Configurations:

F-Type:

- The rippers of a C2 F-type bit engage more material than a C1 F-type.

  - The  C2 F-Type is more aggressive than the C1 F-type.

Mixed Cutlet Type:

- This is an aggressive design similar in nature to the F-type’s aggressiveness but may be more efficient at dislodging and/or fracturing rock.

- The forces applied to the hole act in different directions with different magnitudes of engagement. (aka-simultaneously attack the hole with multiple methods)